TRST-A10 LAN Printer

LAN Setup Utility

User’s Manual

1ST Edition	: Oct 26, 2009

TOSHIBA TEC SINGAPORE PTE LTD

Copyright(C)2009

TOSHIBA TEC SINGAPORE PTE LTD

This document describes the precautions for using the LAN Setup Utility

This document assumes that the reader is familiar with the following:

- General characteristics of Windows and Image Print Driver for Windows

- General characteristics of OPOS for Windows

- General characteristics of TRST-A10-LX1 printer

Notes:

- All rights reserved. It is prohibited to use or duplicate a part or whole of this document without the permission of TOSHIBA TEC SINGAPORE PTE LTD.

- This document is subject to change without prior notice.

  - Microsoft, Windows, Windows NT, Windows 2000, and Windows XP are registered trademarks of Microsoft Corporation in the United States and/or other countries.

  - The official name of Windows is the “Microsoft Windows Operating System”.

Revision Record

No.SPAA-343

TRST-A10 LAN Printer LAN Setup Utility User's Manual

| Rev.No. | Date | Pages | Description |
| --- | --- | --- | --- |
| 01 | Oct. 26, 2009 | --- | Original issue |

Table of Contents

1	Operation Environment	5

2	User I/F	6

2.1	Main Screen	6

2.2	How the menus and controls work	7

2.3	Dialog for Configuration Settings	8

2.4	Dialog About TRST EtherSetup	9

3	Functions	10

3.1	Retrieving printers without “Search Specified Address”	10

3.2	Retrieving printers with “Search Specified Address”	11

3.3	Start Browser	12

4	Appendix	14

4.1	Retrieve option (Examples for Search Specified Address)	14

# Operation Environment

This utility works with below environments.

Supported OS		:  Windows 2000 / XP Professional

Language 		:  English

Available I/F 		:  10Base-T/100Base-TX Ethernet

Protocol		:  UDP/IP

| Table 1.1 Files composing this utility(T.B.D.) |  |
| --- | --- |
| File name | Description |
| TRSTEtherSetup.exe | Application with GUI |
| TRSTEtherSetup.ini | The file to save configuration settings (to search printers) |

# User I/F

## Main Screen

Below is the screen of utility's starting up.

## How the menus and controls work

Configuration(C) is shown when “Tool(T)” is selected.

About(A) is shown when when “Help(H)” is selected.

| Table 2.2 Menus List |  |  |
| --- | --- | --- |
| Menus and Controls |  | Description |
| Tool(T) |  |  |
|  | Configuration(C) | Open modal dialog “Configuration Settings”, if selected (see 2.3 for the details). |
| Help(H) |  |  |
|  | About(A) | Open modal dialog “About TRST EtherSetup”, if selected (see 2.4 for the details). |
| Retrieve |  | Retrieve printers on (specified or default) subnet(s), if clicked, and show their information on the List View (see 3.1 for the details) |
| Browser |  | Launch default browser (defined by OS) with the IP address of selected printer as its start page (see 3.2 for the details) |

## Dialog for Configuration Settings

Below modal dialog is opened when the “Tool(T)”-“Configuration(C)” Menu is selected.

Note:  When “OK” is clicked, below configurations shall be saved in "TRSTEtherSetup.ini”.

1. Enable/Disable of "Search Specified Address".

2. The entry of "IP Address" and "Subnet Mask" in the List View with their entry order.

3. “Timeout(sec)” value.

| Table 2.3 Item List |  |
| --- | --- |
| Controls | Description |
| Search Specified Address | In case this option is enabled, search the entries of "IP Address" and "Subnet Mask" in the List View as well as local network on this utility's retrieval operation.(see 3.2 for the details) |
| IP Address | Input a specific "IP Address" and "Subnet Mask" of the segment to search for. |
| Subnet Mask |  |
| Add Button | Click this to add the "IP Address" and "Subnet Mask" to the "List View". .(see 3.2 for the details) |
| Delete Button | Click this to delete the selected entry(="IP Address" and "Subnet Mask") in the List View. |
| Timeout(sec) | Specify the "Timeout" to receive the printer's response. It's applied for the "Retrieving Printers" operation with Ethernet. Adjustable from 6 to120 and the default is 6(sec). The utility aborts and finishes its communication after this "Timeout" interval. |

## Dialog About TRST EtherSetup

Selecting “Help(H)”-“About(A)” menu to show below modal dialog.

# Functions

## Retrieving printers without “Search Specified Address”

Click “Retrieve” button to search for printers with UDP communication. And the printer’s information will be shown on "List View".

- IP Address

- MAC Address

- Printer model number

- Printer serial number

## Retrieving printers with “Search Specified Address”

Selected the “Tool(T)”-“Configuration(C)” Menu and click the “Search Specified Address” check box.

Please Input a specific "IP Address" and "Subnet Mask" of the segment to search for and then click the “Add” button. (See 4.1 for the details)

After added the IP address and subnet mask, click “OK” button to confirm the setting.

Click the “Retrieve” button and get the printer’s information

## Start Browser

After successful retrieve the printer’s information, clicking the "Browser" button, launch web page provided by the printer selected.

Internet Explore will be pop-up and starting URL is http://xxx.xxx.xxx.xxx

Here, "xxx.xxx.xxx.xxx" is the IP address of the selected printer on the "List View".

# Appendix

## Retrieve option (Examples for Search Specified Address)

| Table 4.1 retrieved printer |  |  |  |
| --- | --- | --- | --- |
| Search Specified Address |  |  | retrieved printer |
| Checked | IP Address | Subnet Mask |  |
| No | - | - | TRST Printer 1(192.168.0.11) |
| Yes | 192.168.1.0 | 255.255.255.0 | TRST Printer 1(192.168.0.11 |
|  | 192.168.1.3 | 255.255.255.255 | TRST Printer 1(192.168.0.11), TRST Printer 2(192.168.1.3) |
|  | 192.168.2.0 | 255.255.255.0 | TRST Printer 1(192.168.0.11) |
|  | 192.168.2.3 | 255.255.255.255 | TRST Printer 1(192.168.0.11), TRST Printer 3(192.168.2.3) |
|  | 192.168.2.4 | 255.255.255.255 | TRST Printer 1(192.168.0.11), TRST Printer 4(192.168.2.4) |
|  | 192.168.2.5 | 255.255.255.255 | TRST Printer 1(192.168.0.11), TRST Printer 5(192.168.2.5) |
|  | 192.168.3.0 | 255.255.255.0 | TRST Printer 1(192.168.0.11) |
|  | 192.168.3.3 | 255.255.255.255 | TRST Printer 1(192.168.0.11), TRST Printer 6(192.168.3.3) |
|  | 192.168.1.0 192.168.2.0 192.168.3.0 | 255.255.255.0 255.255.255.0 255.255.255.0 | TRST Printer 1(192.168.0.11) |
|  | 192.168.1.3 192.168.2.3 192.168.2.4 192.168.2.5 192.168.3.3 | 255.255.255.255 255.255.255.255 255.255.255.255 255.255.255.255 255.255.255.255 | TRST Printer 1(192.168.0.11), TRST Printer 2(192.168.1.3), TRST Printer 3(192.168.2.3), TRST Printer 4(192.168.2.4), TRST Printer 5(192.168.2.5), TRST Printer 6(192.168.3.3) |

*	Assume network setting is correctly configured on HOST PC and each router.
This is out of the specification for "LAN setup Utility".